PROTOCOL FOR 1-STEP RT-qPCR

- Equipment and Consumables Preparation

Sterilization: All glassware should be baked at 180–200°C for at least 4 hours to destroy RNases. Plasticware (tubes, tips) should be certified RNase-free or autoclaved if DEPC-treated water is used.

- TE buffer preparation:

1x TE buffer composition:

| Components | Final concentration | Function |
| --- | --- | --- |
| Tris-HCl | 10mM | Maintains pH |
| EDTA | 1mM | Chelates cations, stabilize nucleic acid |

3. Tris-HCl 1M 50ml (MW = 121.14g)

- Add 6.055g Tris base  to 40ml autoclaved distilled water

- Adjust pH to 8 using HCl

- Make volume upto 50ml

4. EDTA 0.5M 50ml (MW = 372.24g)

- Add 9.306 g disodium EDTA to 35ml double autoclaved water

- Stir well

- Adjust pH to 8 with NaOH. Sometime EDTA don’t dissolve well until pH is 8.

| Stock solution | Volume needed for 10x TE buffer | Volume needed for 1x TE buffer |
| --- | --- | --- |
| 1M Tris-HCl | 5ml | 0.5ml |
| 0.5M EDTA | 1ml | 0.1ml |
| Add required amount of autoclaved distilled water to make volume upto mark |  |  |

5. Preparing TE buffer from solid components

| Components | 10x TE buffer |  | 1x buffer |  |
| --- | --- | --- | --- | --- |
|  | 50ml | 100ml | 50ml | 100ml |
| Tris base | 0.6057g | 1.2114g | 0.06057g | 0.12114g |
| EDTA disodium | 0.186g | 0.372g | 0.0186g | 0.0372g |
| Autoclaved distilled water | Upto 50ml | Upto 100ml | Upto 50ml | Upto 100ml |

- Adjust pH to 8.

6. 1x 10ml TE buffer

1ml 10x TE buffer + 9ml autoclaved distilled water.

7. Primer resuspension

- Spin down the tube in a microcentrifuge to collect all material at the bottom.

- Add 10x ul of 1x TE buffer to the tube’s labelled amount of primer. Ex. If the number on the tube is 30nmol, add 30 x 10 = 300 ul of TE buffer.

- Gently mix the contents by pipetting up and down.

Avoid vigorous mixing to prevent the formation of air bubbles.

- Finally, centrifuge the tube briefly to collect any residual liquid that may be present on the walls of the tube, and then store the resuspended primers at –20°C until further use.

8. Diluting primers: 10 μM 100μl 1st working stock (1:10 Dilution)

- Allow the primer stock solution to thaw on ice.

- Next, gently vortex the primer tube to mix any settled material.

- 10µl of 100 µM primer stock in an empty nuclease free tube. Add 90µl nuclease free water to the tube.

- Pipette up and down to mix.

- Centrifuge the tube briefly to collect any residual liquid that may be present on the walls of the tube.

- Prepare 1:5 dilution to make 2uM 2nd working stock.

- Store the 100uM primary stock at -40°C, 10uM stock at -20°C until further use.

9. Assembling the GoTaq® 1-Step RT-qPCR Reaction Mix

Reaction volume 20μl

- Thaw the GoTaq® qPCR Master Mix and Nuclease-Free Water in ice. Do not thaw the GoTaq® qPCR Master Mix above room temperature.

- Vortex the GoTaq® qPCR Master Mix for 3–5 seconds to mix. Vortex at low speed to avoid aeration.

- Determine the number of reactions to be set up, including negative control reactions. Add 1 reaction to this number to compensate for pipetting error.

- Prepare the reaction mix (minus RNA template) by combining the GoTaq qPCR Master Mix, GoScript  RT Mix, PCR primers and Nuclease-Free Water according to the following chart.

| Component | Volume per 20μl reaction | Volume per 50μl reaction | Final concentration |
| --- | --- | --- | --- |
| qPCR Master Mix, 2x | 10μl | 25 μl | 1x |
| 1-Step RT mix , 50x | 0.4μl | 1μl | 1x |
| Forward primers (10x, assuming 2 uM) | 2 μl | 5μl | 200nM |
| Reverse primers (10x, assuming 2 uM) | 2 μl | 5μl | 200nM |
| RNA template | 4 μl | 10μl | Variable |
| CXR (reference dye) optional | 0.33μl | 0.825 μl | 500nM |
| Nuclease free water | upto 20 μl | upto 50 μl |  |

- Vortex briefly.

- Add 16 ul of reaction mix to each PCR tube.

- Add 4ul RNA template to the appropriate wells of the reaction plate.

- Seal the tubes.

- Centrifuge briefly to collect the contents of the wells at the bottom. Protect from extended light exposure or elevated temperatures.

- Keep the PCR tubes on ice until loading into the thermal cycler.

The samples are ready for thermal cycling.

6. Thermal cycler set-up

| Step | Cycles | Temperature | Time |
| --- | --- | --- | --- |
| Reverse transcription | 1 | 38 | 15 minutes |
| Reverse transcriptase inactivation and DNA polymerase activation | 1 | 95 | 2 minutes |
| Denaturation | 40 | 95 | 10 seconds |
| Annealing and data collection |  | 55 | 30 seconds |
| Extension |  | 72 | 30 seconds |