## General Flow Cytometry Protocol

- Take 20-30 µL of blood in a 10 mL FACS tube and mix well.

- Add 400 µL of lysis buffer (use 200 µL for a 5 mL FACS tube), mix well, and let stand for 5 minutes at room temperature.

- Transfer the sample into an Eppendorf tube and centrifuge at 1600 rpm for 5 minutes at 4°C.

- Ensure all the sample from a single mouse is collected into a single Eppendorf tube through serial centrifugation.

- Wash the sample by adding 800 µL of FACS buffer, centrifuge, and discard the supernatant.

- Add 800 µL of FACS buffer to each tube, mix well, and transfer 400 µL from each tube into an equal amount of new tubes.

- Add 400 µL of FACS buffer to all tubes and mix.

- Add all external antibodies (all CD markers) using 0.5 µL for each, and mix well.

- Let the tubes stand in the dark for 25-30 minutes by placing them on ice.

- Centrifuge the tubes, prepare the tubes designated for intracellular markers, and confirm there is 800 µL of FACS buffer in each tube.

- Centrifuge, discard the supernatant, add 800 µL of permeabilization buffer (composed of 1 µL Tween in 1 mL FACS buffer), and mix well.

- Add the internal antibodies simultaneously and mix well.

- Let the tubes stand in the dark for 25-30 minutes by placing them on ice.

- Centrifuge the tubes, discard the supernatant, and add 800 µL of FACS buffer.

- Mix well and centrifuge.

- Wash the sample with 800 µL of filtered distilled water (DW).

- Add 200 µL of sheath fluid and mix well.

- Run the sample in the CytoFlex machine after a mild vortex.

## Spleen Processing

- Filter 40-50 mL of RBC lysis buffer at room temperature.

- Keep approximately 50 mL of filtered FACS buffer on ice.

- Retrieve the spleen from the -80°C freezer and keep it on ice.

- Prepare a petri dish and a nylon mesh (normal water filter).

- Add 10 mL of FACS buffer into the petri dish.

- Place the spleen on the nylon net.

- Rub the spleen against the net using the piston of a syringe.

- Transfer the resulting solution into a Falcon tube.

- Centrifuge the solution for 5 minutes at 4°C at 4000 rpm.

- Discard the supernatant and retain the precipitate.

- Add 200 µL of RBC lysis buffer and mix.

## Buffer Preparations

### 500 mL FACS Buffer

- Primary Components: 0.18612 g EDTA, 0.5 g NaN3, and 5 mL Fetal Bovine Serum (FBS).

- Alternative Formulation: PBS supplemented with 2% FBS, 1 mM EDTA, and 0.1% Sodium azide.

- Procedure Step 1: Take 50 mL of distilled water (DW) and add the EDTA and NaN3.

- Procedure Step 2: Dissolve the mixture using a magnetic stirrer.

- Procedure Step 3: Transfer the solution into a measuring cylinder.

- Procedure Step 4: Add PBS up to the 400 mL mark, then add the FBS.

- Procedure Step 5: Fill up to the 500 mL mark with PBS, and adjust the pH to 7.4 using NaOH.

### Sheath Fluid (For 5L)

| Reagent | Amount for 5L | Calculation |
| --- | --- | --- |
| NaCl | 39.65 g | (7.93 x 5) |
| Na2EDTA | 1.9 g | (0.38 x 5) |
| KCl | 2.0 g | (0.4 x 5) |
| NaH2PO4 | 0.95 g | (0.19 x 5) |
| Na2HPO4 | 9.75 g | (1.95 x 5) |
| NaF | 1.5 g | (0.3 x 5) |

- Step 1: Take 1 L of DW and add all reagents.

- Step 2: Dissolve the mixture using a glass rod.

- Step 3: Fill up to the 5 L mark with DW and mix.

- Step 4: Adjust the pH to 7.4.

- Step 5: Ensure the sheath fluid is filtered before use.

### RBC Lysis Buffer

- 10X Stock Solution (For 100 mL): Combine 8.02 g NH4Cl, 0.84 g NaHCO3, and 0.37 g EDTA (disodium), then fill up to 100 mL with H2O.

- Dilution Note: The 10X solution must be diluted with distilled water to 1X for working.

- 1X Working Solution: Contains 0.15 M ammonium chloride, 10 mM Potassium bicarbonate, and 0.1 mM EDTA.

### Permeabilization Buffer

- Preparation: Mix 5 mL of FACS buffer with 5 µL of Tween-20, OR 1 mL of FACS buffer with 1 µL of Tween-20.

## Markers & Staining Guidelines

### CD Markers Reference

- CD3: T cells (general).

- CD4: TH cells.

- CD8: Tc cells.

- CD44: Effector cells.

- CD62L: Memory cells / Naive T-cell.

- CD45: Leukocyte.

- Intracellular markers: IFN-γ, IL-4.

### Cellular Staining Requirements

- Cell Suspension: 1 million splenocyte cells should be suspended in 100 µL of pre-cooled FACS buffer.

- Antibody Concentration: Use 0.2 µg of antibodies per 100 µL.

- Final Volume: Cells should ultimately be resuspended in 500 µL of FACS buffer.

- For CD4+ T-lymphocytes: Stain cells in one tube with anti-CD3-PCY5, anti-CD4-Pcy7, anti-CD44-FITC, and anti-CD62L-PE conjugated antibodies.

- For CD8+ T-lymphocytes: Stain cells in a separate tube with anti-CD3-Pcy5, anti-CD8-Pcy7, anti-CD44-FITC, and anti-CD62L-PE conjugated antibodies.

### Flow Cytometry Hierarchy Notes

- CD3+ T Cells: Branches into CD8+ (Tc) and CD4+ (TH) populations.

- CD4+: Relates to IL-4 and IFN-γ expression.

- CD8+: Relates to CD44+/CD62L+ expression (Central memory T cell).

- Immune Response: Tk-dependent cell-mediated immune response.

- RBD Specifics: Acute RBD 154/5417 corresponds to a TH-2 dependent response.

## Software & Machine Maintenance (CytoExpert/CytoFlex)

### Software Experiment Setup

- Software Navigation: Open software > File > New experiment > Name the file "MMI Asma New folder".

- Event Settings: Set Event to 50,000 and Event to record to 50,000.

- Time Settings: Set Time to record to 600 seconds.

- Rate Settings: Set Rate to Medium/Fast.

- Tube Name Settings: Set levels for PE (IFN-γ / IL-4), PC55 (CD3), and PC7 (CD4/CD8).

- Execution: Proceed to Record, followed by Analysis.

### Machine Maintenance Procedures

- Deep Clean Protocol: Power on the machine > Open CytoExpert > Run deep clean for 30 minutes > Prime.

- Daily Clean Protocol: Power on the machine > Open CytoExpert > Initialize > Select Daily clean > Run with cleaning liquid for 3 minutes > Run with filtered DDW for 2 minutes > Proceed to Experiment.